ChEn 285 Lab Set Up

Pump Cart Orientation

Equipment Needed for 1 Station:

- Pump Cart

There are 4 pump carts available.

Other Equipment Needed:

- Hoses for filling and draining tank

Chemicals Needed:

- None

To Set Up Station:

- Wheel the pump cart to the desired location

- Engage brakes on 2 wheels closest to handle

- Use a wrench to lower the pneumatic lifts enough to firmly touch the floor but not raise the cart. This will support the weight of the water-filled tank

- Ensure that the drain valve is closed

- Attach a short hose to the water line from the wall

- Place the free end of the hose into the tank and fill about 2/3 full (adding too much water increases the weight and leads to splashing)

- Disconnect hose and store in the labeled cabinet

- Connect ethernet cable from cart to wall port (all ethernet ports are activated). This must be done before connecting the power so the cart will register the proper IP address

- Connect 110 V power cable from cart to wall outlet

- Connect 220 V power cable from cart to wall outlet (twist lock)

- Connect the compressed air (yellow coiled hose) from the cart to the quick connect on the wall. Open the compressed air valve until the gauge reads about 60 psi

- To control the flow with the vi accessed from the Fundamentals website, no need to plug computer Ethernet into the wall, just use the wifi.

To Take Down Station:

- Close the wall valve on the compressed air and remove the yellow hose from the quick connect on the wall

- Unplug the 220 V power cable from the wall outlet

- Unplug the 120 V power cable from the wall outlet

- Unplug the ethernet cable from the wall port

- Secure all cables by wrapping them around the electrical box at the top of the cart so they don’t drag on the floor

- Connect the hose to the outlet of the tank and place the free end over the drain in the floor

- Open the drain valve and drain all of the water

- Close the drain valve, remove hose, and store hose in labeled cabinet

- Use a step stool to reach into the tank and wipe it out with a paper towel, especially the bottom surface. This will prevent build up

- Use a wrench to raise the pneumatic lifts until they are as high as possible. You want to maximize the clearance for transporting the cart

- Push the cart back to the CB for storage