ChEn 285 Lab Set Up

Viscometer

Equipment Needed:

- 2-4L glass beaker

- 2-Lachoi Hot plate w/ temperature probe

- 2-Stir bar

- 4-Viscometer

- 4-Ring stand

- 4-Three-prong clamp

- 4-Three-way bulb w/ tubing

You need two stations with all the equipment above and two stations with the last four items above (viscometer, ring stand, three-prong clamp, three-way bulb w/ tubing).

Other Equipment:

- Scale

- Small beakers

Chemicals Need:

- DI water

- Ethanol

To Set Up Station (start set up at least one hour before lab):

- Place two Lachoi hot plate w/ temperature probes on the table and plug into outlet.

- Place one ring stand next to each hot plate with a three-prong clamp attached to each stand.

- Fill the two 4L glass beakers with water ½” from the top.

- Place the two 4L glass beakers onto the two hot plates.

- Drop the stir bar into each of the beakers.

- Turn the heater on and heat to 40°C while stirring with the stir bar at a minimum of 200 rpm.

- Place the empty viscometer in the clamp.

- Place the three-way bulb w/ tubing next to the set up.

- This portion is ready for the students.

- For the last two set ups, set up two rings stand viscometer stations without the water bath or hot plate.

- All stations are ready for the students.

- Fill and label a small beaker with ethanol (used to charge the viscometer and weigh for density).

- Fill and label another small beaker with DI water (used to charge the viscometer and weigh for density).

- Ensure a scale is available to be used to weigh the ethanol and DI water.

To Clean up Station:

- Turn off the hot plates and allow to cool to safe temperature.

- Once cooled, remove the stir bar and dump water into drain.

- Put the hot plate, stir bar, and 4L beaker away.

- Remove viscometers from clamps.

- Dump any remaining liquid from the viscometers in an appropriate waste container.

- Use ethanol to remove any remaining residue inside the viscometers.

- Blow air into the bulbs to dry using the three-way bulb w/ tubing

- Return viscometers and three-way-bulb w/ tubing to appropriate storing location.

- Return ring stand and three-prong clamp to appropriate storing location.